Annotated reference list with methodological details regarding the calculation of values in the dams, rivers & canals databases.

ADB (2011)

Monthly mean flows for the Kaveri were taken from Figure 56, p. 84 of ADB (2011) and averaged over the period 2001-2011, which is the period covered by the report. Data comes from the Upper Anicut gauging station, also known as Upper Anaicut and Mokkombu Dam. These data were used to calculate both the mean annual discharge in Table 4 and the mean monthly discharge values of Fig.4 and Table 5.

Bihar State Hydroelectric Power Corporation Limited (BSHPCL), Indrapuri Reservoir Project (450 MW): Salient Features, http://www.bshpcltd.com/Indrapuri.HTM, Accessed 09 10 2016 (2007).

Chaube, U. C., Suryavanshi, S., Nurzaman, L., and Pandey, Ashishi, Synthesis of flow series of tributaries in Upper Betwa Basin, Int. J. Environ. Stud. 1(7), 1459-1475 (2011).

Mean annual discharge data for the Betwa comes from Table 6, observed discharge at Rajghat. Although the table caption gives the units as MCM, I believe this is a typo – units of m3s-1 -are consistent with other sources for the Betwa.

Center for International Earth Science Information Network (CIESIN), Columbia University, Gridded Population of the World, Version 4 (GPWv4): Population Density. Palisades, NY: NASA Socioeconomic Data and Applications Center (SEDAC). http://dx.doi.org/10.7927/H46T0JKB. Accessed 21 Apr 2016 (2015).

Central Water Commission (CWC), Feasibility report of Sapta Kosi High Dam Project (1981). http://environmentclearance.nic.in/writereaddata/Online/TOR/0_0_21_Jul_2015_1109178131Additionalattachment.pdf, Accessed 09 10 2016.

Central Water Commission (CWC), Integrated Hydrological Data Book (Non-classified River Basins), www.cwc.nic.in/main/downloads/IHD2015_final.pdf, Accessed 10/09/2016 (2015).

The 2015 Integrated Hydrological Data Book was used to calculate mean annual discharge for four rivers at their terminal gauging stations, using annual totals compiled by the Central Water Commission. For each river, the following data were used. (1) Penna at Nellore station, 2002-2012 with 2006-2007 missing. Table 5 Part VIII, pg. 197.  (2) Achankovil at Thumpamon station, 2009-2012. Table 5 Part IX, pg. 202. (3) Pamba: data from Malakkara station, 2009-2012. Table 5 part IX, pg. 201. (4) Netravati: data from Bantwal station, 2002-2012 with 2007-2009 missing. Table 5 part IX, pg. 200. Although longer periods of record were available for some rivers (e.g., Achankovil median annual discharge 1977-2012, Table 7 Part IX, pg. 255), the 2002-2012 data were selected to better represent current conditions of the rivers.

de Condappa, D., Barron, J., Tomer, S. K., and Muddu, S. Impact assessment of agricultural water management interventions in the Jaldhaka watershed, Stockholm Environmental Institute Project Report 2011 (2012).

Mean annual discharge for the Jaldhak (Jaldhaka) river was calculated using de Condappa et al. (2012)’s modeled average streamflow for the period 1998-2008 (2342 mm, Table 7, p. 15) divided by the basin area (5795 km3, Table 1, p. 3).

Dhungel, D. N. and Pun, S. B., The Nepal-India Water Relationship: Challenges, Dordrecht, The Netherlands: Springer (2009).

Sapta Gandaki Dam height/storage capacity/submergence area from Dhungel & Pun (2009), Table 3.3 pg. 80, which is itself compiled from Water and Energy Commission Secretariat (WECS) of Nepal Bulletin 7(1), December 1995.

Everard, M. and Kataria, G., The proposed Pancheswar dam, India/Nepal: a preliminary ecosystem services assessment of likely outcomes. London, UK: Institution of Environmental Sciences (2010).

Expert Appraisal Committee (EAC), Minutes of the 71st meeting of the Expert Appraisal Committee for River Valley and Hydroelectric Projects constituted under the provisions of EIA Notification 2006, 20-21 Jan 2014, New Delhi, India, 89 pp. Accessed 09 10 2016, http://environmentclearance.nic.in/writereaddata/Form-1A/Minutes/0_0_311412561216172ndMinutesofEACmeeting.pdf (2014).

Gourdji, S., Knowlton, C., and Platt, K., Indian Inter-linking of Rivers: A preliminary evaluation, Unpublished Master’s Project, University of Michigan, Ann Arbor, MI School, City, State (2005).

Gourdji et al. (2005) present charts obtained from the NWDA during a trip to India in 2004 (Appendix III, pgs. 115-117). These charts give exact transfer volumes for links affecting the Ganges and Brahmaputra basin, which are not otherwise available due to the classification of NWDA Feasibility Reports for these basins. Values from these charts were used in the canals database. The charts also give 75% dependable yield for the rivers – this value was used for the Torsa discharge, as no mean annual or 50% dependable yield (median) value could be located. 75% dependable yield corresponds to the 1st quartile of discharge.

Gourdji, S., Knowlton, C., Platt, K., and Wiley, M. J., Chapter 7: Modeling the Interlinking of the Ganges River: Simulated Changes in Flow, In Mizra, M. M. Q., Ahmed, A. U., and Ahmad, Q. K, (Eds.), Interlinking of Rivers in India: Issues and Concerns, 107-127, London, UK: Taylor & Francis (2008)., doi: 10.1201/9780203894576.ch7.

Gourdji et al. (2008) use the values from the Gourdji et al. (2005) charts to perform a similar analysis to this study for the total change in flow at Farakka Barrage in the Ganga basin given full implementation of the NRLP. Since 2008, changes in the plans for diversions from the Gandak and Ken rivers have been published. Our study uses these updated values; an additional change is to examine discharge at Hardinge Bridge rather than at Farakka Barrage, so that diversions to the Hooghly are also included. We note that otherwise, our values for total river changes match those of Gourdji et al. (2008) for the Ganga basin tributaries.

Husain, S. M., Interlinking of rivers – status, issues and challenges, Presentation of S. Masood Husain, Director General of the National Water Development Agency, to the Jal Manthan-2 Conference, 22-23 Feb 2016, New Delhi, India, Accessed 09 10 2016 (2016).
http://wrmin.nic.in/writereaddata/1-2-1_ILR%20STATUS,%20ISSUES%20AND%20CHALLENGES.pdf.

Institute of Water Modeling (IWM), Ganges river basin modelling: final report, Report number 103885, Washington, D.C.: World Bank Group (2010).

Because the Ganga basin is classified, exact measurements of discharge for the Ganga and its tributaries are not available. Therefore, we have primarily relied on estimates by the Institute of Water Modeling for the current mean annual discharge of Ganga tributaries. Values in this study come from IWM (2010), Fig. 5.58, pg. 100 – Ganges basin flows in base scenario. The base scenario in IWM (2010) covers the period 1998-2007. Discharge values in IWM (2010) are simulated using MIKE BASIN, a model that represents the hydrology of a basin including rainfall-runoff processes, snowmelt, and reservoirs and dams. Existing infrastructure including irrigation and urban water use, dams, and hydropower reservoirs are included in the IWM (2010) analysis for the base scenario. We note that IWM (2010) investigates the impacts of the NRLP in their analysis of Scenario H; values in Figs. 5.60 and 5.61 (pgs. 102 and 103) can be considered equivalent to the “change” values in this study for the Ganges tributaries, but for the months of April and August rather than annually. Similarly, Fig 5.53 (pg. 97) is directly comparable to Hardinge Bridge mean monthly discharge in this study. However, not all diversions and water usage considered in this study were included in IWM (2010).

Jain, S. K., Agarwal, P. K., and Singh, V. P., Hydrology and Water Resources of India, Water Sci. Technol. LIbr. 57, Dordrecht, The Netherlands: Springer (2007).

Chapter 9, Fig. 2 (pg. 423) was used for the mean annual discharge of the Manas and Sankosh rivers.

Jian, J., Webster, P. J., and Hoyos, C. D., Large-scale controls on Ganges and Brahmaputra river discharge on intraseasonal and seasonal time-scales, Q. J. R. Meteorol. Soc. 135, 353-370 (2009)., doi: 10.1002/qj.384.

Madhya Pradesh Pollution Control Board (MPPCB) (2013) www.mppcb.nic.in/pdf/Public_Hearing-sum/mohanpura-exe-eng.pdf, accessed 09 10 2016.

Mehmood, K., Patel, A., Singh, V., Prajapati, S., Kalubarme, M. H., Prakash, I., and Gupta., K. P., Submergence analysis using geo-informatics technology for proposed dam reservoirs of Par-Tapi-Narmada River Link Project, Gujarat State, India, Int. J. Geosci. 5, 622-633 (2014). doi:10.4236/ijg.2014.56057.

Ministry of Environment, Forest and Climate Change (MoEFCC), Executive summary report of CEIA study for Mohanpura Irrigation Project, Forest Clearance File No. 8-02/2014-FC: Kundaliya Major Multipurpose Project Fact Sheet (2014), http://forestsclearance.nic.in/search.aspx?cat_id=FP/MP/IRRIG/3931/2013, Accessed 09 10 2016.

Panda, D. K., Kumar, A., Ghosh, S., and Mohanty, R. K., Streamflow trends in the Mahanadi River basin (India): Linkages to tropical climate variability, J. Hydrol. 495, 135-149 (2013)., doi: 10.1016/j.jhydrol.2013.04.054.

The data in Table 3, p. 141 of Panda et al. (2013) was used as the mean annual discharge of the Mahanadi.. Panda et al. (2013) gives mean annual discharge over the period 1972-2005 at Tikerpara gauging station at the basin outlet. While Panda et al. (2013) points out that the discharge at Tikerpara has experienced an overall decreasing trend during this period, we have chosen to keep the value over the longer period as the decrease in discharge is only 11 m3 s-1 per year.

Press Information Bureau (PIB), Rehabilitation Issues of Polavaram Dam Project – Press Release, 01 December 2011, http://www.pib.nic.in/newsite/erelease.aspx?relid=78016, accessed 09 10 2016 (2011).

Press Information Bureau (PIB),Polavaram Project – Press Release, 17 July 2014, http://pib.nic.in/newsite/PrintRelease.aspx?relid=106830%20, accessed 09 10 2016 (2014).

South Asia Network on Dams, Rivers & People (SANDRP), Interlinking of Rivers in India: Dimensions of Social Impacts, SANDRP Report DRR-0107, January 2007 (2007). http://sandrp.in/riverlinking/DRR_paper_0107.pdf, Accessed 09 10 2016.

South Asia Network on Dams, Rivers & People (SANDRP), TOR & environment clearance status of river valley projects: project wise details: April 2007 to Dec 2012, Accessed 09 10 2016 (2013). http://sandrp.in/env_governance/EAC_meetings_Decisions_All_India_Apr_2007_to_Dec_2012.pdf.

Srivastava, M., Bishwapriya, A.,Kumar, S. V., and Kumar, A., Report on feasibility stage geotechnical investigations for Gandak-Ganga link canal project, Uttar Pradesh, Engineering Geology & Geotechnical Investigation NRO_21452 of the Geological Survey of India, Accessed 04 11 2016 (2006). http://www.portal.gsi.gov.in/gsiDoc/pub/cs_gandak-ganga1.pdf.

Tehri Hydro Development Corporation (THDC) India Limited, Sankosh H.E. Purpose Project – Features, http://thdc.gov.in/Projects/english/Scripts/Prj_Features.aspx?Vid=171 (2012). Accessed – 09 10 2016.

Tiwari, H., and Sharma, N. (2013), Empirical models for sediment and discharge of Raidak River, Novus Int. J. Eng. Tech. 2(3), 11-15.

Mean discharge for the Raidak from Table 2, p. 13 of Tiwari & Sharma (2013).

Sabha, R. (2014), Government of India Ministry of Water Resources, River Development and Ganga Rejuvenation Memo – Question No. 124, 07 07 2014, Accessed 09 10 2016, http://indiaenvironmentportal.org.in/content/395796/question-raised-in-rajya-sabha-on-inter-linking-of-rivers-07072014/

Singh, D. B. (2013), Storage and pumped storage projects: Demand of the nation, Presentation of Dilli Bahadur Singh, Director General of the Department of Electricity Development, to the Hydropower Investment Meeting, 28-29 Aug 2013, Kathmandu, Nepal, http://www.him2013.hidcl.org.np/pdf/Storage-Pumped-Storage-Project-DB-Singh.pdf, Accessed 09 10 2016.

World Bank (1992), Karnali Preparation Project – Phase I, Project Completion Report – No. 11013, Energy and Infrastructure Operations Division, country Department I, South Asia Regional Office, World Bank, 34 pp. http://documents.worldbank.org/curated/en/971271468289863838/pdf/multi-page.pdf, Accessed 09 10 2016.

Part III: National Water Development Agency (NWDA) Sources

The following NWDA documents have been archived and analyzed. Note that dates of publication do not reflect dates of access; all documents were downloaded in 2015 and 2016. In this study it has been assumed that an NWDA document available for access from the main NWDA portal (nwda.gov.in) is still relevant; if a more recent source contradicts the information in the NWDA documents, this has been noted in the database and included only if the source is reliable (e.g., a geotechnical investigation, or a memo or presentation by NWDA officials). Dates of publication by NWDA may not reflect dates of data collection; many reports contain data from surveys and censuses conducted as early as the 1980s. For displaced population estimates, dates of census were used in this study rather than dates of publication. For all river discharge values, date ranges of data collection are specified in the database.

Header gives canal # (this study) followed by NWDA name

1.1 – 1.5 / Manas-Sankosh-Tista-Ganga

- In-house bulletin (NWDA, 2013)

2 / Kosi-Mechi

- Executive Summary (NWDA, 2009)

- Detailed Project Report (NWDA, 2009)

6.1-6.2 / Sarda Yamuna

- Prefeasibility report (NWDA, 2015)

9.1-9.3b / Parbati Kalisindh Chambal

- Summary (NWDA, 2005)

- Feasibility Report (NWDA, 2005)

10 / Ken-Betwa

- Summary (NWDA, 1995)

- Feasibility Report (NWDA, 1995)

- Updates to Feasibility Report (NWDA, 2009)

- Detailed Project Report Phase I (NWDA, 2011)

- Detailed Project Report Phase II (NWDA, 2013)

- Environmental Impact Assessment and Environmental Management Plan for Phase I (NWDA, 2015)

16.1-16.2 / Mahanadi (Manibhadra)-Godavari (Dowlaiswaram)

- Summary (NWDA, 2015)

17 / Godavari (Polavaram)-Krishna (Vijayawada)

- Summary (NWDA, 1995)

- Feasibility Report (NWDA, 1995)

18 / Godavari (Inchampalli) Krishna (Pulichintila)

- Summary (NWDA, 2005)

- Feasibility Report (NWDA, 2005)

19 / Godavari (Inchampalli)-Krishna (Nagarjunasagar)

- Summary (NWDA, 2005)

- Feasibility Report (NWDA, 2005)

20 / Krishna (Nagarjunasagar)-Pennar (Somasila)

- Summary (NWDA, 1999)

- Feasibility Report (NWDA, 1999)

21 / Pennar (Somasila)-Palar-Cauvery (Grand Anicut)

- Summary (NWDA, 2005)

- Feasibility Report (NWDA, 2005)

22 / Krishna (Srisailam)-Pennar

- Summary (NWDA, 1999)

- Feasibility Report (NWDA, 1999)

23 / Krishna (Almatti)-Pennar

- Summary (NWDA, 2005)

- Feasibility Report (NWDA, 2005)

24 / Bedti-Varada

- Summary (NWDA, 1995)

- Preliminary Feasibility Report (NWDA, 1995)

25 / Netravati-Hemavati

- Summary (NWDA, 2005)

26 / Cauvery (Kattalai)-Vaigai-Gundar

- Summary (NWDA, 2005)

- Feasibility Report (NWDA, 2005)

27.1-27.2 / Pamba-Achankovil-Vaippar

- Summary (NWDA, 1995)

- Feasibility Report (NWDA, 1995)

28.1-28.4 / Par-Tapi-Narmada

- Summary (NWDA, 2005)

- Feasibility Report (NWDA, 2005)

- Detailed Project Report (NWDA, 2012)

29.1-29.2 / Damanganga-Pinjal

- Feasibility Report (NWDA, 2005)

- Detailed Project Report (Pinjal Dam) (NWDA, 2012)